| Function Description | Torque Sharing |  |  |
| --- | --- | --- | --- |
| Applicable Models | C2000 | File version | Ver 00 |
| Applicable Firmware | C2000 : Above Ver. 2.01 | Date | 2019/11/12 |

- Content Description

Torque sharing function is designed for multi-drive applications, where the system is driven by multiple C2000s, and the motor shafts are coupled with each other through gears, chains, or conveyor belts. The system has the following features：

- Achieve torque sharing between master and slave, load can be evenly distributed among the drive units.

- The external control PLC signal is only connected to the master, and the master controls the slave through the internal PLC of the inverter.

- The master is a typical speed control, and the slave follows the output torque of the master :

- When the motor shafts of the master and slave are rigidly connected by gears, chains, etc., the control mode of the slave is recommended to adopt torque control, so that there is no speed difference between the transmission units.

- When the motor shafts of the master and slave are flexibly connected, the control mode of the slave is recommended to adopt speed control, because a slight speed difference between the transmission units is allowed.

- 系統架構

- Relative Devices :

  - HMI and PLC: According to customer selection.

  - VFD: C2000 series, support up to 8 slaves.

  - Motor: FOCPG supports synchronous motors and induction motors, FOC Sensor-less supports induction motors.

  - Built-in PLC program.

- The core values：

- Internal PLC with torque sharing function based on inverter, no need to add additional master-slave controller.

- The internal communication RS485 has a scan cycle of 4ms and supports a system architecture of one master and eight slaves at most.

- Communication disconnection self-detection to improve system stability.

- Perfect protection mechanism, when one inverter fails, the system will stop running.

- Based on current vector control algorithm, improve motor control accuracy.

- Parameter adjustment SOP

A.  The VFD on the Master side is set to FOCPG or FOC sensor-less mode.

- Set nameplate parameters from motor

- If the load cannot be disconnected: Set 2 for induction motor Pr[05-00] to perform auto-tuning, and set 13 for synchronous motor Pr[05-00] to perform auto-tuning.

- If the load can be disconnected: Set 6 for induction motor Pr[05-00] to perform auto-tuning, and set 5 for synchronous motor Pr[05-00] to perform auto-tuning.

- Set to torque sharing function of master drive mode :

- Manual setting, please refer to Table 1 for setting parameters (FOCPG refer to Table 3).

- Or set Pr[07-28] to 2, let the VFD automatically set the parameters according to Table 1.

- Set Pr[04-60], select the slave to start up.

- Set the operating frequency and direction of the master（Pr[04-50] 、Pr[04-51]）.

- Slave inverter is set to TQCPG or TQC sensor-less mode

- Set nameplate parameters from motor

- If the load cannot be disconnected: Set 2 for induction motor Pr[05-00] to perform auto-tuning, and set 13 for synchronous motor Pr[05-00] to perform auto-tuning.

- If the load can be disconnected: Set 6 for induction motor Pr[05-00] to perform auto-tuning, and set 5 for synchronous motor Pr[05-00] to perform auto-tuning.

- Set to torque sharing function of slave drive mode :

- Manual setting, please refer to Table 2 for setting parameters (FOCPG refer to Table 4).

- Or set Pr[07-28] to 3, let the VFD automatically set the parameters according to Table 2.

- Set Pr[09-31] slave station number.

Table 1, Master parameter setting (FOC sensor-less mode)

| Parameters | Setting | Description |
| --- | --- | --- |
| Pr[00-10] | 0 | Speed mode |
| Pr[00-11] | 5 | FOC sensor-less |
| Pr[04-50] | 20.00 | System frequency command |
| Pr[04-51] | 0 | Direction of rotation, 0: forward; 1: reverse |
| Pr[04-52] | 0.00 | Starting frequency of the slave |
| Pr[04-60] | 1 | Enable VFD slave |
| Pr[10-24] bit8 | 0 | Speed feedback selection, 0: observer; 1: PG Card |
| Pr[11-00] bit0 | 1 | ASR gain automatic adjustment |
| Pr[09-31] | -10 | master mode |

Table 2, Slave parameter setting (TQC sensor-less mode)

| Parameters | Setting | Description |
| --- | --- | --- |
| Pr[00-10] | 2 | Torque mode |
| Pr[00-13] | 2 | TQC sensor-less |
| Pr[00-20] | 1 | Speed command by RS485 |
| Pr[00-21] | 2 | Operation command by RS485 |
| Pr[04-60] | 1.00 | Speed error 1.00Hz (positive limit) |
| Pr[04-61] | 1.00 | Speed error 1.00Hz (negative limit) |
| Pr[04-62] | 0.00 | ASR Droop Rate |
| Pr[04-63] | 1.000 | Transmission ratio with master and slave |
| Pr[09-02] | 2 | Internal communication disconnection processing method |
| Pr[09-31] | -1~-8 | slave mode (station number 1~8) |
| Pr[10-24] bit5 | 1 | Turn on ADD speed limit mode |
| Pr[10-24] bit7 | 1 | Turn off TQC low-speed flux adjustment function |
| Pr[10-24] bit8 | 0 | Speed feedback selection, 0: observer; 1: PG Card |
| Pr[11-00] bit0 | 1 | ASR gain automatic adjustment |
| Pr[11-33] | 1 | Torque command by RS485 |

Table 3, Master parameter setting (FOC PG mode)

| Parameters | Setting | Description |
| --- | --- | --- |
| Pr[00-10] | 0 | Speed mode |
| Pr[00-11] | 3 | FOCPG |
| Pr[04-50] | 20.00 | System frequency command |
| Pr[04-51] | 0 | Direction of rotation, 0: forward; 1: reverse |
| Pr[04-52] | 0.00 | Starting frequency of the slave |
| Pr[04-60] | 1 | Enable VFD slave |
| Pr[10-24] bit8 | 1 | Speed feedback selection, 0: observer; 1: PG Card |
| Pr[11-00] bit0 | 1 | ASR gain automatic adjustment |
| Pr[09-31] | -10 | master mode |

Table 4, Slave parameter setting (TQC PG mode)

| Parameters | Setting | Description |
| --- | --- | --- |
| Pr[00-10] | 2 | Torque mode |
| Pr[00-13] | 0 | TQCPG |
| Pr[00-20] | 1 | Speed command by RS485 |
| Pr[00-21] | 2 | Operation command by RS485 |
| Pr[04-60] | 1.00 | Speed error 1.00Hz (positive limit) |
| Pr[04-61] | 1.00 | Speed error 1.00Hz (negative limit) |
| Pr[04-62] | 0.00 | ASR Droop Rate |
| Pr[04-63] | 1.000 | Transmission ratio with master and slave |
| Pr[09-02] | 2 | Internal communication disconnection processing method |
| Pr[09-31] | -1~-8 | slave mode (station number 1~8) |
| Pr[10-24] bit5 | 1 | Turn on ADD speed limit mode |
| Pr[10-24] bit8 | 0 | Speed feedback selection, 0: observer; 1: PG Card |
| Pr[11-00] bit0 | 1 | ASR gain automatic adjustment |
| Pr[11-33] | 1 | Torque command by RS485 |

- Application

- Pipe Jacker :

- Pipe jacking construction is to install a pipe jacker at the front end of the pipe, and break the soil into fluid soil or mud through the front of the pipe jack to push the pipeline forward

- Equipment structure :

- Control System Configuration :

- Material feeder :

- The material feeder is an auxiliary equipment for transportation machinery. As the purpose of uniform feeding of ore, it can also be used for short-distance transportation of materials with a large proportion; heavy-duty plate feeders are widely used in mining, metallurgy, building materials, ports, coal and industrial and mining enterprises

- Equipment structure :

- Control System Configuration :

- Note

- Before performing motor parameter identification, please confirm whether the load can be disconnected, and then select static or dynamic parameter tuning.

- Since the synchronous motor Sensor-less has a lower load capacity at low speeds, the synchronous motor FOC Sensor-less torque sharing function is temporarily not supported

- When adjusting, please test the single running performance of the master and the slave separately, confirm the single running performance and steering are correct, and then perform the load linkage for system test